| Vermont Lead Paint Law | Shaded items are more restrictive | EPA Renovation, Repair & Painting Rule |
| --- | --- | --- |
| * Pre-1978 rental housing and child care facilities must complete EMP's * LSWP required in pre-1978 owner occupied | Applicability | Pre-1978 housing and child occupied facilities, including schools, etc. |
| LSWP required if more than 1 SF of paint to be disturbed, either interior or exterior | Threshold/ Trigger | LSWP required if more than 6 SF disturbed interior; 20 SF disturbed exterior |
| EMP certification only (no cost) | License | Renovation Firms licensed by EPA ($300) |
| * 4-hour EMP Training - one time (generally free) * 1 trained supervisor per job site | Training | * 8-hour EPA initial training * 4-hour EPA refresher every 5 yrs. * 1 trained supervisor per job site |
| Not required | Pre-Work Notification | * Provide owners/occupants with Renovate Right pamphlet, info about work to be completed. * Keep records of notification * Requirement in place since 1998 |
| * Copies of EMP Compliance Statements provided to tenants when submitted to VDH * Protect Your Family pamphlet provided to tenants at lease-up and with compliance statements annually *State specific requirements for sale or transfer of pre-1978 residential prop. | Disclosure | * Since 1996, owners required to disclose all records of LBP activities to potential buyers and/or tenants. * Standard form, pamphlet provided to buyers/tenants. * Keep all records of disclosure activities. |
| *Visual inspections for paint deterioration * Prompt repairs using LSWP * Window well inserts * Poster to report problems * Specialized cleaning at turnover * Specialized cleaning after work * Removal of exterior paint chips | Property Maintenance | Not required - LSWP by licensed renovator only when paint is disturbed or repaired over threshold amount. |
| In addition to EPA prohibited practices: * dry scraping / dry sanding * all machine sanding or grinding * uncontained power washing * strippers containing methylene chloride | Prohibited Practices | * open flame burning/torching * power sanding or grinding without HEPA exhaust controls * heat guns over 1100 degrees |
| * Visual check after all cleaning * No visible dust and debris | Clearance | * EPA cleaning verification procedure * 3rd party dust clearance & lab analysis in some situations |
| * Completed EMP Compliance Statements provided to residents when submitted | Post-Work Notification | * Provide records of activities and cleaning verification to owner and occupants |
| * Compliance Statements * Records of providing Compliance Statements and pamphlets to tenants, others. * Records of inspections/work done | Recordkeeping | * Renovation firms must maintain all records, sign-offs, etc. for 3 years after project completion. * Property owners must keep all records and disclose them to appropriate parties as noted above. |